Study Report from the Vietnam National University of Forestry

(Némethi Levente, Jakab András)

Introduction

This report summarizes our experiences and everything we learned during a one-month

exchange program at the Vietnam National University of Forestry (VNUF). Over the past

weeks, we had the unique opportunity to see firsthand how Vietnam successfully blends

traditional woodworking with modern, large-scale manufacturing. From designing furniture

with CAD/CAM software and learning about bamboo processing, to visiting local artisan

workshops, this journey was incredibly hands-on and eye-opening. The following sections

highlight our most important practical and academic takeaways from this month.

1

. Furniture Design and CAD/CAM Integration - Hien Nguyen Thi Thanh

The initial phase of the academic program was dedicated to modern furniture engineering,

emphasizing the integration of Computer-Aided Design (CAD) and Computer-Aided

Manufacturing (CAM) technologies. Practical training focused on structural 3D modeling

utilizing SketchUp software, complemented by advanced toolpath generation and CNC

programming using Vectric Aspire. These digital methodologies are essential in contemporary

wood processing for optimizing material yield and ensuring high-precision manufacturing.

During the practical sessions, comprehensive structural designs were developed for three

distinct cabinetry typologies. The first project involved the spatial planning, proportioning,

and structural detailing of a standard two-door wardrobe. This was followed by a more

complex architectural integration task: designing a six-door built-in storage unit, which

required precise spatial optimization and alignment calculations to meet interior architectural

constraints.

The last design assignment was a multifunctional, asymmetric cabinet configuration. This

complex unit was engineered to include three basal drawers, a primary full-length vertical

door, and two superior doors situated above the drawer section. These practical exercises

facilitated a deep understanding of digital modeling, component assembly detailing, and the

preparation of digital files for automated CNC machining processes within the wood industry.

2

. Material Science and Industrial Utilization of Bamboo - Phuoc Le ngoc

The curriculum also explored the structural properties and industrial processing of

Dendrocalamus barbatus (Luồng bamboo). While noted for its high density and thick walls,

raw bamboo presents natural physical limitations—such as a small diameter, a hollow stem,

and a large taper—which complicate direct utilization. To mitigate biological degradation,

targeted harvesting, borate impregnation, and rigorous kiln-drying are applied.

To overcome structural limitations, modern industrial utilization involves breaking bamboo

into elementary units and recomposing them via adhesives into large-scale, high-strength

panels. The primary manufacturing methodologies studied included:

•

•

•

•

Strip processing: Breaking the culm into flattened or sawn-shaved strips.

Sliver processing: Producing thin slivers to weave into mats or curtains.

Particle processing: Converting residues into particles or fibers for composite boards.

Composite processing: Integrating bamboo units with other materials (e.g., wood,

paper, plastics) to optimize integral properties. These principles were practically

observed in the manufacturing of Laminated Bamboo Panels (LBP), which involves

milling raw slats, starch-leaching via hydrothermal treatments, and high-pressure hot

pressing with low-VOC adhesives.

3

. The Vietnamese Wood Industry and Furniture Export - Hung Phan Duy

Vietnam has established a highly competitive furniture export sector driven by substantial

forestry resources and a skilled, cost-effective labor force. The nation's total forest area

reaches nearly 15 million hectares with a 42% cover rate, including 4.89 million hectares of

planted forests and 1.1 billion cubic meters of timber reserves. Despite this abundance,

domestic harvesting meets only 77.4% of the industry's demand, requiring 22.6% of timber to

be imported to sustain production. To address this deficit, the sector is aiming for self-reliance

while adopting foreign technological advancements to boost its $19 billion market capacity.

Furthermore, the industry benefits from a diligent population of over 100 million, where 69%

of the workforce possesses vocational training and 29.5% hold higher professional

qualifications. Labor costs remain highly competitive globally, averaging $250 to $350 per

month, which significantly bolsters export margins. This strategic resource management and

qualified workforce have propelled the industrial GDP contribution from 5% in 1986 to

2

4.5% in 2025, with a projected target of 28% by 2030.

4

. Growth Characteristics and Veneer Utilization of Fast-Growing Plantation

Species - Mai Trinh Hien

Recent research presented on fast-growing plantation species—specifically Acacia hybrid,

Acacia mangium, and Eucalyptus—highlighted their morphological development and

industrial processing characteristics. Studies on their growth dynamics indicate rapid cross-

sectional and height increments; however, stem cylindricity and form quality can vary

significantly. Notably, the Acacia hybrid generally exhibits superior cylindrical form and

straightness compared to pure A. mangium.ꢀUtilizing advanced hydrothermal pre-treatments

and rotary peeling technologies, they yield cost-effective, high-quality veneers that are

extensively processed into plywood and Engineered Timber Products (ETPs), thereby

ensuring a sustainable raw material supply for the sector.

5

. Vietnamese Timber Species and Their Architectural Applications - Nga

Hoang Thi Thuy

The timber utilized in the construction and furniture industries is primarily divided into two

main categories: high-strength natural hardwoods and sustainable plantation species.

•

Natural Hardwoods: Natural forests covering over 10.2 million hectares provide

highly durable timber with a density ranging from 0.7 to 1.2 g/cm³ and a service life

exceeding 100 years. Exceptionally heavy and load-bearing species such as Lim

(Ironwood), Táu, and Sến offer excellent termite resistance and are widely used for

traditional houses, temples, bridges, and heavy structural columns. Conversely, Gõ Đỏ

(Redwood) and Căm Xe (Burmese Rosewood) are celebrated for their attractive dense

grain and dimensional stability, dominating luxury interiors, premium furniture, and

flooring.

•

•

Plantation Timber: Covering approximately 4.7 to 5.0 million hectares, plantation

timber serves as the backbone of the modern wood industry. Teak wood, featuring a

high natural oil content, grants outstanding weather and moisture resistance, making it

an ideal material for outdoor decking and facades. Pine is a lightweight softwood

commonly chosen for interior ceilings and lightweight cabin architecture. Acacia and

Rubberwood provide an economical and sustainable source of raw materials for

interior finishes, furniture, and engineered wood panels (Plywood, MDF).

Bamboo and Engineered Timber Products (ETPs): Vietnam is home to over 200

bamboo species. Large-diameter species like Luồng and Tầm Vông are extensively

applied in constructing structural frames and pavilions due to their high strength-to-

weight ratio. The future of sustainable construction is heavily driven by the growth of

Engineered Timber Products (such as LVL, Glulam, CLT, and Bamboo Scrimber),

which provide large-span capabilities, high structural efficiency, and fast installation

for mass timber and modular buildings.

6

. Chemical Modification and Combustion Behavior of Fast-Growing Woods -

Tham Nguyen Thi

To address the low density, poor mechanical strength,

and high fire susceptibility of fast-growing plantation

woods like poplar (Populus adenopodaMaxim.) and

radiata pine (Pinus radiata Don.), chemical

modifications utilizing a combination of styrene-

acrylic copolymer (SAC) and sodium silicate (SS) are

applied. The combustion behavior and thermal

stability of these treated woods reveal several critical

industrial insights:

•

Ignition Resistance: Combined treatment with

SAC and SS significantly increases the

difficulty of ignition, resulting in a longer time

to ignition (TTI) and a higher limiting oxygen

index (LOI).

•

•

Structural Charring: Sodium silicate deposits

within the cell lumina (vessels and tracheids)

and forms a thermally insulating glassy

protective film, promoting increased residual char formation during pyrolysis.

Smoke and Toxicity Hazards: Despite the delayed flaming and increased ignition

difficulty, the overall fire risk is not mitigated. The incorporation of SS alongside SAC

ultimately leads to higher total heat release, dense smoke production, and elevated

carbon monoxide (CO) concentrations, which present an increased danger regarding

casualties in fire scenarios.

7

. Surface Finishing Materials and Panel Overlays - Hong Pham Thi Anh

In the context of the global wood and furniture industry, the selection of appropriate surface

overlays is critical for balancing aesthetics, durability, and cost-efficiency. The primary

materials utilized include:

•

Wood Veneer: Extracted via slicing or peeling (0.2–

3.0 mm thickness), 100% natural wood veneers

(e.g., oak, ash, teak) offer eco-friendly aesthetics at

a lower cost than solid wood. However, they require

post-processing (oiling) and are susceptible to

moisture, heat, and mechanical damage, making

them suitable primarily for premium furniture and interior MDF/HDF applications.

Melamine Paper: Comprising decorative paper

impregnated with melamine resin (0.1–0.2 mm

•

•

thickness), this highly cost-effective material provides

versatile patterns for mass-produced cabinetry. Despite

its visual flexibility, it remains sensitive to moisture

and cannot be easily repaired.

Acrylic Panels (PMMA): Ranging from 0.7 to 1.4 mm

in thickness, these synthetic plastics offer premium, UV-resistant

high-gloss or matte finishes. Although expensive and prone to

fingerprints, their flawless glass-like effect makes them highly

desirable for modern kitchen cabinetry, requiring high-precision

equipment and PU adhesives for application onto HDF/MDF

substrates.

•

Laminated Panels (HPL/CPL): Constructed from multiple layers of

phenolic-impregnated kraft paper topped with melamine-

infused decorative overlays (typically 6 layers, 0.5–1.0 mm

thick). They provide exceptional mechanical, heat, and water

resistance, rendering them ideal for high-traffic countertops,

despite physical limitations in sharp-angle bending.

8

. Field Observations of Termite Wood Degradation - Phuong Tong Thi and

Loan Nguyen Thi

During our field excursions, we observed firsthand the devastating impact of termite colonies

on local timber. These insects completely hollow out standing tree trunks, reducing the solid

wood fibers into a spongy, reddish-brown, soil-like mass. Furthermore, this severe mechanical

degradation is often accompanied by secondary fungal infections, visible as white mycelial

growth on the decaying wood. Witnessing this structural collapse in natural environments

perfectly illustrated why the rigorous chemical preservation and modification treatments

discussed earlier are absolutely critical for tropical wood products.

9

. Vacuum Press Technology and Surface Laminating - Dai Vu Huy

The session explored the operational principles of Programmable Logic Controller (PLC)

driven vacuum presses, which ensure uniform pressure distribution across complex

geometries. Specific applications were highlighted, including the precise veneering and

laminating of curved and concave surfaces, as well as the seamless application of colored

decorative foils over irregular 3D profiles. This presentation effectively demonstrated the

technology's critical role in modern custom furniture manufacturing and complex architectural

joinery.

1

0. Microscopic Wood Anatomy – Kien Nguyen Trong

As for the wood anatomy studying we used an inovative, portable optical setup: a clip-on

smartphone microscope providing a 200x base magnification, which we further amplified

utilizing the mobile device's integrated digital zoom. This technique allowed for the rigorous

morphological examination of hardwood specimens, such as the Trám trắng sample evaluated

during our sessions. By analyzing transverse cross-sections, we could accurately observe the

distribution, grouping, and density of porous structures and vessel elements within the wood

matrix. At peak magnification, intricate cellular features became distinctly visible, allowing

for the study of complex fiber arrangements and what appear to be detailed perforation plates.

Furthermore, this imaging technique enabled us to document critical mechanical stress points

and structural anomalies within the samples, such as prominent radial cracking.

The smartphone's digital capture facilitated precise, real-time data collection of both

longitudinal and transverse structural variations. Ultimately, this hands-on wood anatomy

work demonstrated the viability of mobile optical tools in rigorous botanical analysis.

11. Field Observations of Local Manufacturing and Handicrafts - Dai Vu Huy

and Phuong Tong Thi

Field excursions to local processing facilities revealed the dynamic duality of Vietnam's wood

and bamboo industries, where advanced technology and traditional craftsmanship seamlessly

coexist. We observed primary log processing, automated CNC machining, and precision

cutting alongside highly skilled manual labor, including intricate wood carving and the hand-

painting of statues. Furthermore, visits to bamboo and rattan weaving workshops showcased

the intricate production of high-value handicrafts, such as woven baskets and lampshades.

This integration of modern mechanized production with heritage artisanal techniques

highlights the versatility, cultural richness, and economic resilience of the local manufacturing

sector.

Conclusion

In conclusion, our month at VNUF was an amazing experience that went far beyond the

classroom. We had the chance to see the entire spectrum of the wood industry—from

examining wood anatomy under a smartphone microscope to understanding how Vietnamese

furniture reaches the global market. Witnessing the perfect mix of proud, traditional

craftsmanship and modern, sustainable technologies was truly inspiring. This program clearly

showed us that the future of the wood and furniture industry lies in respecting our natural

resources while boldly embracing new engineering solutions.